ESP32 P4C5 Hardware and Camera Research

Task 1  |  12 October 2026  |  Documentation review

This report provides source links, compares six development boards, and assesses three cameras for an ESP32-P4 platform. OV2710 is the preferred first sensor when its module wiring matches the carrier. IMX219 has a community implementation; the HBVCAM camera is a conditional USB UVC option. These are integration assessments, not results from testing the supplied hardware.

Hardware identification. The camera models are OV2710-15P-PCB-V1, Raspberry Pi Camera Module V2 with IMX219, and HBVCAM F2170HD V66. The last model corrects the F210HD V66 label in the task brief. ESP32-P4C5 identifies a P4 plus C5 platform; it does not establish a universal carrier-board pinout.

1 Direct links to hardware documentation

Board documentation below is manufacturer or official material. Schematic revisions must match the physical board. PDF file pages on GitHub provide a Download raw file option.

B1 Espressif ESP32 P4X C5 Function EV Board

Official P4+C5 reference board. The schematic is hardware revision 2.0.

B1a  Official user guide
https://docs.espressif.com/projects/esp-dev-kits/en/latest/esp32p4/esp32-p4x-c5-function-ev-board/user_guide.html

B1b  Official schematic PDF
https://dl.espressif.com/schematics/ESP32_P4X_C5_Function_EV_board-2.0-schematics.pdf

B2 OSPTEK ESP32 P4C5 Module Dev Board

Manufacturer documentation. The linked carrier schematic is V1.3; it is not proof of a match to another V1.2 PCB.

B2a  English user guide PDF file page
https://github.com/osptek/esp32-p4c5-module-dev-board/blob/main/docs/ESP32-P4-Module_User%20Guide%2020260910.pdf

B2b  Carrier schematic V1.3 PDF
https://raw.githubusercontent.com/osptek/esp32-p4c5-module-dev-board/main/docs/ESP32P4%E6%A8%A1%E7%BB%84%E5%9F%BA%E7%A1%80%E5%BA%95%E6%9D%BFV1.3.pdf

B2c  Manufacturer board resources and revision history
https://github.com/osptek/esp32-p4c5-module-dev-board

B3 Wireless Tag WTDKP4C5 S1

Manufacturer documentation. N16R16 and N16R32 are distinct memory configurations.

B3a  User guide PDF
https://res.8ms.xyz/wiki_assets/WTDKP4C5-S1/WTDKP4C5-S1_%E4%BD%BF%E7%94%A8%E6%8C%87%E5%8D%97_V1.1.pdf

B3b  Board schematic PDF
https://res.8ms.xyz/wiki_assets/WTDKP4C5-S1/WTDKP4C5-S1-1V3_SCH_V1.1.pdf

B3c  Core module datasheet PDF
https://res.8ms.xyz/wiki_assets/WTDKP4C5-S1/WT01P4C5-S1_%E6%8A%80%E6%9C%AF%E8%A7%84%E6%A0%BC%E4%B9%A6_V1.1.pdf

B4 Camemake CM ESP P4 C5

Manufacturer repository includes design archives and a board specification.

B4a  Open hardware repository
https://github.com/Camemake/CM_ESP_P4_C5-Open-Hardware-Platform

B4b  Board specification V1.0 PDF file page
https://github.com/Camemake/CM_ESP_P4_C5-Open-Hardware-Platform/blob/main/CM_ESP32_P4_C5_V1.0%20Spec%20Sheet.pdf

1 Direct links to hardware documentation continued

C1 OV2710 15P PCB V1

The sensor brief confirms MIPI and DVP capabilities. The OSPTEK example is a MIPI module reference; an exact PCB-V1 schematic and 15-pin map remain unverified.

C1a  OmniVision sensor product brief mirrored by Espressif
https://dl.espressif.com/AE/esp-dev-kits/ov2710pbv1.1web.pdf

C1b  OSPTEK OV2710 MIPI module and ESP32 P4 example
https://github.com/osptek/camera-mipi-csi-ov2710

C2 Raspberry Pi Camera Module V2 IMX219

Official module documentation. The schematic is for the Raspberry Pi Camera V2.1 PCB.

C2a  Official module schematic PDF
https://datasheets.raspberrypi.com/camera/camera-module-2-schematics.pdf

C2b  Official product specification
https://www.raspberrypi.com/products/camera-module-v2/

C3 HBVCAM F2170HD V66

The closest official family specification located is V22. Its OV2720 sensor, UVC formats, frame rates and power figures must not be assumed for V66.

C3  Manufacturer F2170HD V22 family reference
https://www.hbvcamera.com/2-Mega-%20Pixel-usb-cameras/hbvcam-ov2720-2mp-1080p-auto-focus-camera-module-for-industrial-equipment.html

Additional board and software references

B5  Espressif P4X Function EV Board official user guide with C6 radio
https://docs.espressif.com/projects/esp-dev-kits/en/latest/esp32p4/esp32-p4x-function-ev-board/user_guide.html

B6  Waveshare ESP32 P4 WIFI6 official user guide
https://docs.waveshare.com/ESP32-P4-WIFI6

B6b  Waveshare board schematic PDF
https://files.waveshare.com/wiki/ESP32-P4-WIFI6/ESP32-P4-WIFI6-datasheet.pdf

D1  Espressif esp_cam_sensor 2.6.0 support matrix
https://components.espressif.com/components/espressif/esp_cam_sensor/versions/2.6.0/readme

D2  Community esp_cam_sensor_imx 0.4.0 author documentation
https://components.espressif.com/components/mushbraindave/esp_cam_sensor_imx/versions/0.4.0/readme

D3  Espressif usb_host_uvc 2.6.0 driver documentation
https://components.espressif.com/components/espressif/usb_host_uvc/versions/2.6.0/readme

2 Comparison of six development boards

All six boards use an ESP32-P4 dual-core RISC-V application processor with a low-power RISC-V core and a separate wireless companion. Clock limits depend on silicon and board revision. Memory below is PSRAM / Flash on the P4 side. Fit statements are engineering assessments; connector compatibility requires a separate pinout check.

| Board and sources | PSRAM / Flash | Wireless | Camera and display | Price and availability | Similarity and difference |
| --- | --- | --- | --- | --- | --- |
| Espressif ESP32-P4X-C5-Function-EV-Board [B1a B1b] | 32 / 16 MB | C5 2.4/5 GHz Wi-Fi 6 BLE | MIPI CSI MIPI DSI USB HS Host | Official purchase channel; price and stock unverified. | Closest official P4+C5 reference. Also includes Ethernet and audio. |
| OSPTEK ESP32-P4C5-Module Dev Board [B2a B2b] | 32 / 16 MB | C5 2.4/5 GHz Wi-Fi 6 BLE | CSI 24P/15P DSI 30P/15P USB-A Host | Vendor store links; price and stock unverified. | Same chip pairing, with multiple connector formats. Carrier revision affects control-signal wiring. |
| Wireless-Tag WTDKP4C5-S1 [B3a B3c] | 16 or 32 / 16 MB Select SKU | C5 2.4/5 GHz Wi-Fi 6 BLE | MIPI CSI MIPI DSI USB HS OTG | Vendor purchase link; price and stock unverified. | Same architecture. Verify N16R16 versus N16R32 and the fitted module revision. |
| Camemake CM_ESP_P4_C5 [B4a B4b] | 32 / 16 MB | C5 2.4/5 GHz Wi-Fi 6 BLE | MIPI CSI MIPI DSI Parallel display | Public design archives; kit price and stock unverified. | Same P4+C5 class. Uses the Camemake camera connector definition; check each sensor and driver. |
| Espressif ESP32-P4X-Function-EV-Board [B5] | 32 / 16 MB | C6 2.4 GHz Wi-Fi 6 BLE | 15P MIPI CSI 15P MIPI DSI USB HS Host | Official sample reference US$56 [P1]; package and stock vary. | Similar multimedia functions; C6 has no 5 GHz Wi-Fi. P4X uses silicon revision 3.1 or later. |
| Waveshare ESP32-P4-WIFI6 [B6 B6b] | 32 / 32 MB | C6 2.4 GHz Wi-Fi 6 BLE | 2-lane CSI MIPI DSI USB HS 4Pin | Official indexed listing US$13.99-69.99 [P2]; includes different kits. | More Flash and a C6 radio. USB cameras require the correct Host cable and VBUS power. |

Price basis. The US$56 figure is an official sample reference, not a verified checkout total. The Waveshare range spans board and kit options and was visible in the official indexed listing; live stock was not confirmed. Compare equivalent package contents, shipping and taxes before purchase.

P1  Espressif official development board catalogue and sample reference price
https://www.espressif.com/en/products/devkits

P2  Waveshare official product listing and kit options
https://www.waveshare.com/product/esp32-p4-wifi6.htm

Selection. A documented P4+C5 board is the closest replacement when 5 GHz Wi-Fi is required. P4+C6 boards offer a similar camera/display pipeline when 2.4 GHz Wi-Fi is sufficient. The actual target carrier remains unidentified, so none of these schematics is a verified wiring diagram for it.

3 Camera interfaces and feasibility on ESP32 P4

Feasibility depends on the module interface, carrier wiring, driver and capture pipeline. A matching FFC pin count alone does not establish electrical compatibility.

| Camera | Interface | Assessment |
| --- | --- | --- |
| OV2710-15P-PCB-V1 | Module is a MIPI candidate; sensor also supports DVP. | Conditionally usable; preferred first sensor after confirming module wiring. |
| Raspberry Pi Camera Module V2 IMX219 | Two-data-lane MIPI CSI-2. | Conditionally usable with a community driver and compatible carrier. |
| HBVCAM F2170HD V66 | USB; UVC compliance and modes need enumeration. | Conditionally usable through a powered P4 USB Host port. |

OV2710 15P PCB V1

The OV2710 sensor supports one MIPI data lane, parallel DVP and RAW output. Espressif esp_cam_sensor 2.6.0 includes an OV2710 MIPI driver, and OSPTEK publishes a P4 preview example. This gives a credible implementation path, but does not verify the supplied PCB. Confirm its 15-pin map, supply input, SCCB/I2C, clock, reset and power-down signals before connection. A passive ribbon adapter cannot convert DVP into CSI. RAW frames also need an appropriate ISP and encoding path. The official driver matrix marks OV2710 as discontinued. [C1a]  [C1b]  [D1]

Raspberry Pi Camera Module V2 IMX219

The official V2.1 module schematic exposes two CSI-2 data lanes, a clock pair, I2C, enable and 3.3 V input. The module has local regulators and a 24 MHz oscillator. Generic IMX219 support is absent from the checked Espressif 2.6.0 sensor matrix, but the community esp_cam_sensor_imx 0.4.0 author reports hardware operation on Waveshare ESP32-P4-WIFI6 with ESP-IDF 5.4.0. Begin with the author-recommended 1640 x 1232 mode. Check driver dependencies, lane rate, ISP settings and cable mapping; full 8 MP processing and operation on the target board are not established. [C2a]  [D1]  [D2]

HBVCAM F2170HD V66

The supplied camera uses USB. The official F2170HD V22 page describes UVC operation, MJPEG/YUY2 and 5 V power, but those specifications do not establish V66 behaviour. If V66 enumerates as a compatible UVC camera, Espressif usb_host_uvc 2.6.0 provides a P4 path supporting bulk and isochronous streams. Use a real USB Host port with VBUS power, not a CSI connector or a UART-only debug port. Read VID/PID, descriptors, endpoints and supported frame modes. If available, start with a modest MJPEG mode; do not assume 1080p operation or infer the camera-side pin order from wire colours. [C3]  [D3]

Remaining checks and implementation order

First identify the target carrier and verify the OV2710 module pinout, then try the official OV2710 MIPI path. If that wiring cannot be established and a powered USB Host port is available, enumerate the HBVCAM camera as an alternative. Keep IMX219 as a community-driver option. Connector pitch, contact side, polarity, supply and control voltage must be checked for the chosen board. No supplied device was connected, flashed or electrically tested for this report.

AI links

：https://chatgpt.com/s/cx_6accc4940d5481918cf41827bfec6371

However, I couldn't find the specific model details for the ESP32 v66, so I used the v22 as a reference; similarly, I relied on general information for the OV2710, as I couldn't locate a guide specifically tailored to the hardware version I have on hand.